- General Description

TFT1.8 LCD Serial Module is based on SPI serial port of 1.8Inch TFT LCD, compatible with 5V and 3.3V power supply, which can be used to display Chinese and English, pictures and color screen etc...

Please note：The module apply 5V power supply acquiescently, if you want to change to 3.3V, power supply changeover from 5V/3.3V to 3.3V on PCB is needed by soldering tools.

Features

- Power supply 3.3 V/5V

- Resolution：128*160

- 5PIN SPI USB interface

- Size 58.00mm×34.50mm

- Applications

- Chinese and English, pictures and color screen display

- Work with Arduino modules to be the part of Arduino kit.

- Electronic Characteristics

| PARAMETER | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | - | 5/3.3 | - | V |

- Pining Information

(1)

| Types | Symbols | Description |
| --- | --- | --- |
| LCD Serial Module PIN | VCC | POWER Supply 5V/3V3 |
|  | GND | POWER Ground |
|  | CS | Chip Selection，Low level active |
|  | RST | Reset，Low level active |
|  | AO | Data/Command selection |
|  | SDA | Serial data input |
|  | SCK | Serial clock input |
|  | LED | BACK LED POWER Supply 5V/3V3 |

- Interface Description

(2)

- Dimension Outline

(3)

- Revision History

| REVISION | DESCRIPTION | RELEASE DATE |
| --- | --- | --- |
| V1.0 | Initial version | 07/09/2015 |

- Contact Information

More information please refer to: http://www.elecfreaks.com